githubcodematlab

Emergency Vehicle Siren Detection — MATLAB Source Code

Instructions: Save the code below as a MATLAB script (for example, githubcodematlab.m). Ensure the audio file is in the Current Folder or one of the searched folders. Change INPUT_FILE to test a different recording.

%% EMERGENCY SIREN DETECTION - RUN ONE AUDIO FILE AT A TIME

% First run with traffic_noise_siren.wav (traffic + siren).

% Then change only INPUT_FILE below to traffic_noise_only.wav and Run again.

% All analysis plots are generated for whichever file is selected.

% Requires Signal Processing Toolbox.

clc; clear; close all;

%% CHANGE THIS ONE LINE EACH TIME

INPUT_FILE = 'Noisy_Siren.wav'; % Test 1: traffic + siren

% For Test 2, replace the line above with:

% INPUT_FILE = 'Non_Siren_Test.wav';  % traffic/noise only

LOW_HZ = 500;

HIGH_HZ = 1300;

POWER_RATIO_THRESHOLD = 0.15;

fprintf('\n==============================================\n');

fprintf(' EMERGENCY VEHICLE SIREN DETECTION\n');

fprintf('==============================================\n');

fprintf('Input audio: %s\n', INPUT_FILE);

% Find the selected file in Current Folder or project dsp.

folders = {pwd, fullfile(pwd,'project dsp'), ...

'/MATLAB Drive/project dsp', '/MATLAB Drive'};

audioPath = '';

for k = 1:numel(folders)

candidate = fullfile(folders{k}, INPUT_FILE);

if exist(candidate,'file') == 2

audioPath = candidate;

break;

end

end

if isempty(audioPath)

error('Cannot find %s. Check INPUT_FILE and the folder.', INPUT_FILE);

end

%% STEP 1 - LOAD AUDIO

[x, Fs] = audioread(audioPath);

if isempty(x), error('The selected audio file is empty.'); end

if size(x,2) > 1, x = mean(x,2); end

x = x(:) - mean(x(:));

if Fs <= 2*HIGH_HZ

error('Sampling rate must be greater than 2600 Hz for this filter band.');

end

t = (0:length(x)-1)'/Fs;

duration = length(x)/Fs;

fprintf('\nSTEP 1: Loaded audio | Fs = %d Hz | Duration = %.2f s\n',Fs,duration);

figure('Name','Step 1 - Input Waveform','Color','w');

plot(t,x); grid on; xlabel('Time (s)'); ylabel('Amplitude');

title(['Input Audio Waveform: ' INPUT_FILE]);

%% STEP 2 - TIME-DOMAIN DETAIL

figure('Name','Step 2 - Waveform Detail','Color','w');

nShow = min(length(x), round(2*Fs));

plot(t(1:nShow),x(1:nShow)); grid on;

xlabel('Time (s)'); ylabel('Amplitude');

title('Input Audio - First 2 Seconds');

%% STEP 3 - BUTTERWORTH BAND-PASS FILTER

[b,a] = butter(4,[LOW_HZ HIGH_HZ]/(Fs/2),'bandpass');

xf = filtfilt(b,a,x);

fprintf('STEP 3: Applied 4th-order Butterworth band-pass filter (%d-%d Hz)\n',LOW_HZ,HIGH_HZ);

figure('Name','Step 3 - Filtered Waveform','Color','w');

plot(t,xf); grid on; xlabel('Time (s)'); ylabel('Amplitude');

title('Band-Pass Filter Output (500-1300 Hz)');

[h,fh] = freqz(b,a,2048,Fs);

figure('Name','Step 3 - Filter Response','Color','w');

plot(fh,20*log10(abs(h)+eps)); grid on; xlim([0 min(3000,Fs/2)]);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)');

title('Butterworth Band-Pass Filter Response');

%% STEP 4 - FFT OF INPUT

N = 2^nextpow2(length(x));

X = abs(fft(x,N));

f = (0:N/2)'*Fs/N;

X = X(1:N/2+1); X(1)=0;

[~,iFFT] = max(X);

fftPeak = f(iFFT);

fprintf('STEP 4: FFT peak frequency = %.2f Hz\n',fftPeak);

figure('Name','Step 4 - Input FFT','Color','w');

plot(f,X); grid on; xlim([0 min(4000,Fs/2)]);

xlabel('Frequency (Hz)'); ylabel('Magnitude');

title('FFT Spectrum of Input Audio');

%% STEP 5 - FFT OF FILTERED AUDIO

Nf = 2^nextpow2(length(xf));

XF = abs(fft(xf,Nf));

ff = (0:Nf/2)'*Fs/Nf;

XF = XF(1:Nf/2+1); XF(1)=0;

figure('Name','Step 5 - Filtered FFT','Color','w');

plot(ff,XF); grid on; xlim([0 min(3000,Fs/2)]);

xlabel('Frequency (Hz)'); ylabel('Magnitude');

title('FFT Spectrum After Filtering');

%% STEP 6 - SPECTROGRAM

figure('Name','Step 6 - Spectrogram','Color','w');

spectrogram(x,hamming(400),300,1024,Fs,'yaxis');

ylim([0 min(3,Fs/2000)]); title('Input Audio Spectrogram');

%% STEP 7 - WELCH POWER SPECTRAL DENSITY

winLength = min(1024,max(64,floor(length(x)/4)));

[Pxx,fw] = pwelch(x,hamming(winLength),floor(winLength/2),...

max(2048,2^nextpow2(winLength)),Fs);

[~,iWelch] = max(Pxx);

peakHz = fw(iWelch);

figure('Name','Step 7 - Welch PSD','Color','w');

plot(fw,10*log10(Pxx+eps),'LineWidth',1.1); grid on; hold on;

xline(LOW_HZ,'--r','500 Hz'); xline(HIGH_HZ,'--r','1300 Hz');

xlim([0 min(4000,Fs/2)]);

xlabel('Frequency (Hz)'); ylabel('PSD (dB/Hz)');

title('Welch Power Spectral Density');

%% STEP 8 - SIREN BAND-POWER RATIO

band = fw>=LOW_HZ & fw<=HIGH_HZ;

df = mean(diff(fw));

bandPower = sum(Pxx(band))*df;

totalPower = sum(Pxx)*df;

ratio = bandPower/(totalPower+eps);

fprintf('STEP 8: Welch peak = %.2f Hz | Siren-band ratio = %.4f\n',peakHz,ratio);

figure('Name','Step 8 - Band Power Comparison','Color','w');

bar([bandPower, max(totalPower-bandPower,0)]);

set(gca,'XTickLabel',{'Siren band 500-1300 Hz','Outside siren band'});

ylabel('Estimated Power'); grid on;

title('Power Inside vs Outside Expected Siren Band');

%% STEP 9 - FINAL DETECTION

% This is a simple frequency-and-power detector. It analyzes the signal,

% not the filename. Threshold may need tuning for different recordings.

detected = peakHz>=LOW_HZ && peakHz<=HIGH_HZ && ratio>=POWER_RATIO_THRESHOLD;

if detected

decision = 'SIREN DETECTED';

decisionColor = [0 0.5 0];

else

decision = 'NO SIREN DETECTED';

decisionColor = [0.8 0 0];

end

fprintf('\n==============================================\n');

fprintf('FINAL RESULT: %s\n',decision);

fprintf('File: %s\n',INPUT_FILE);

fprintf('Peak frequency: %.2f Hz\n',peakHz);

fprintf('Siren-band power ratio: %.4f\n',ratio);

fprintf('==============================================\n');

figure('Name','FINAL DETECTION RESULT','Color','w','Position',[200 200 850 350]);

axis off;

text(0.5,0.65,decision,'Units','normalized','HorizontalAlignment','center',...

'FontSize',30,'FontWeight','bold','Color',decisionColor);

text(0.5,0.40,['Audio file: ' INPUT_FILE],'Units','normalized',...

'HorizontalAlignment','center','FontSize',16);

text(0.5,0.22,sprintf('Peak %.1f Hz | Band-power ratio %.3f',peakHz,ratio),...

'Units','normalized','HorizontalAlignment','center','FontSize',14);

title('SIREN DETECTION OUTPUT','FontSize',18);

%% STEP 10 - SAVE MEASUREMENTS

Result = table({INPUT_FILE},Fs,duration,fftPeak,peakHz,ratio,{decision},...

'VariableNames',{'FileName','SamplingRate_Hz','Duration_s','FFTPeak_Hz',...

'WelchPeak_Hz','BandPowerRatio','Decision'});

writetable(Result,'Siren_Detection_Result.csv');

fprintf('Saved measurements to Siren_Detection_Result.csv\n');